# 737-200A LIMITATIONS

## STARTER LIMITS									EGT LIMITS					    -15A		   -17A

°C				°C

ON		OFF		CYCLES					START GROUND				575			575

30”			60”				3						START AIR						620			650

or														MCT								580			610

60”			60”				2						TAKE-OFF			5 MIN		620			650

or														ACCELERATION	2 MIN		630			660

90”			  5’

30”			  5’										FLT START IGNITION CYCLE:	2’ ON	  3’ OFF

2’ ON	23’ OFF

## STARTER MOTORING							ENGINE OIL PRESSURE

ON		OFF		CYCLES					MAX			55 psi

90”			90”				2						MIN			40 psi

then			15’

### ENGINE RPM										ENGINE OIL TEMPERATURE

MAX N1		102.4%							MAX CONTINUOUS	130°C

MAX N2		100.0%							MAX 15 MIN				130-165°C

MAX						165°C

### OPERATIONAL

MAX OPERATING ALTITUDE						37,000 FT

MAX WIND FOR AIRSTAIR OPERATION		40 kts

MAX R/W SLOPE									±2%

MAX TWC FOR LANDING/TAKE-OFF			15 kts

MAX TURBULENT PENETRATION SPEED	M0.70 / 280 KIAS

MAX SPEED WITH MACH TRIM INOP			M0.74

TAKE-OFF WITH ANTI-SKID INOP IS NOT CERTIFIED.

MAX STRUCTURAL TAKE-OFF WEIGHT						52,0 ton

MAX ZERO FUEL WEIGHT										43,1 ton

MAX LANDING     WEIGHT										48,5 ton

### PRESSURIZATION

OPERATING	DIFF PRESSURE			7.8 ±0.1 psi

MAX			DIFF PRESSURE			8.650 psi

MAX CABIN DIFF FOR T/O & LNDG		0.125 psi

### PNEUMATICS

MAX EXTERNAL AIR PRESSURE	60 psi @ 450°F  (232°C)

APU

MAX APU OPERATING ALTITUDE			35,000 ft

MAX CONT 	EGT		710°C

MAX 			EGT		760°C

APU GENERATOR LIMIT	GROUND			125 AMPS

APU GENERATOR LIMIT	INFLIGHT		111 AMPS

MAX

APU BLEED + ELECTRICAL LOAD			10,000 ft

APU BLEED	ONLY								17,000 ft

APU ELECTRICAL LOAD ONLY				35,000 ft

APU BLEED VALVE MUST BE CLOSED WHEN:

- GROUND AIR CONNECTED & ISOLATION VALVE IS OPEN

- ENGINE #1 BLEED VALVE IS OPEN

- ENGINE #2 BLEED VALVE & ISOLATION VALVE IS OPEN

- ENGINE THRUST ABOVE IDLE DURING ENGINE START

### ICE & RAIN PROTECTION

MIN N1 RPM FOR OPERATING IN ICING CONDITIONS EXCEPT FOR LANDING:

TAT	0-10°C		40%

TAT	   <0°C		55%

TAT	<6.5°C		70%	IN MODERATE TO SEVERE ICING CONDITIONS

### FUEL

MIN TEMP			+03°C	(fuel freezing point)

MAX TEMP			+49°C

MAX FUEL QUANTITY:		CENTRE	TANKS	7,417 kg

MAIN		TANKS	4,588 kg

MAX MAIN TK IMBALANCE:

Taxi, Take-off & Flight:			680 kg

Landing:							590 kg

Main Tank 2

MIN FUEL FOR GROUND OPERATION OF HYD SYSTEM B PUMPS:			760 kg

## ELECTRICAL

MAX ENGINE GENERATOR LOAD		111 AMPS

MAX CSD OIL TEMP						157°C

## FLIGHT CONTROLS

MAX FLAP EXTENSION ALTITUDE		20,000 ft

MIN ALT FOR SPEEDBRAKE USAGE	     500 ft

ALTERNATE FLAP DUTY CYCLE:		1 CYCLE ON THEN 25 MIN OFF